AWSP Clear WaterSlide Decal Paper will allow image transfer to objects that cannot be heated with a heat press.

It is recommended for application to:

White or Light Colored Candles, Glass, Metal, Wood, Soap

Items required:  Computer, Color Laser Printer, Large Water bowl, Dish Soap, Plain Paper, Plastic bag or plastic wrap.

Step 1

Feed transfer paper into laser printer so that the printer is printing on the printable side (white glossy coated side). It is recommended to use the bypass tray when printing.  Note  The printable side is the white glossy coated side.

Step 2

Design your image and resize if necessary. Do not mirror image the design

Step 3

Trim the imaged Water Slide paper as close as possible without cutting into image using scissors / exacto knife.

Step 4

Fill suitable sized bowl with water and add about 5-10% liquid dish soap.

Step 5

Place a piece of plastic wrap on the counter or work surface. Put a little water from the bowl on the plastic wrap.  Then place the imaged paper in bowl for 5-10 seconds.

Step 6

Place the wet decal paper from the bowl on the pre-wet plastic bag/wrap face down. Absorb access water with a plain piece of paper.

Step 7

Gently remove paper backing from plastic bag/wrap. Place the imaged plastic bag/wrap on top of material to be printed.  With sponge and finger, gently correct the position of image and smooth out air bubbles.

Step 8

Let the transfer and material dry for at least 1 hour before handling.

Tips:  For extra durability on candles, using a glue stick will help adhere the Decal paper.

The candle may also be heated with a blow dryer to soften the wax before applying the transfer.

This paper will work in most Laser Printers.  Testing is highly recommended

1500 Greenleaf Avenue, Elk Grove Village, Illinois 60007  •   Local 847-981-0386  •  Toll Free 800-882-0386  •  Fax 847-981-1353

www.awpapers.com